Whole and Trace Element Geochemistry Summary of altered and mineralized samples of the Kurupung Batholith.

In an effort to document geochemical signatures and variations across the batholith and mineralized zones, selected sample suite was submitted to ActLabs for research grade whole rock, trace element, and REE geochemical analyses.  The samples submitted ranged from unaltered Kurupung into K-metasomatized domains through to the albite alteration associated with the uranium mineralization.  This summary focuses on the geochemical signatures of the alkaline signatures of the alteration zone associated with late stage K-Na metasomatism.  The following samples were selected to capture: (1) the initiation of K-spar growth; (2) transition into K-metasomatism with blocky K-spar over the metamorphic fabric; (3) initiation of Na-metasomatism with white albite rinds on K-spar; (4) complete albitization defined by blocky red albite; and (5) intense zircon and U-mineraralization.

| ARN094 102.3 | Minor secondary K-spar growth over Kurupung | Initiation of K-spar growth |
| --- | --- | --- |
| ARW030 208.8 | Relatively fresh Kurupung with minor secondary K-spar growth | Initiation of K-spar growth |
| ARN048 65.5 | Blocky feldspar over fabric | Transition into K-metasomatism |
| ARS029 83.5 | Blocky feldspar over fabric | Transition into K-metasomatism |
| ARS094 151.80 | Secondary K-spar | Transition into K-metasomatism |
| ARS090 209.25 | White rinds on K-spar | Initiation of Na-metasomatism |
| ARS043 154 | Rinding of K-spar by albite | Initiation of Na-metasomatism |
| ACCORI NORTH C 011 123.0 | Albitized cataclasite | Complete albitization |
| ARW008 106.8 | Blocky red albite | Complete albitization |
| SAMPLE C1 (COGEMA CORE) | Albitite | Complete albitization |
| SAMPLE C5 (COGEMA CORE) | Albitite | Complete albitization-Zr-U-mineralization |

U3O8 Corp., routinely submit mineralized and altered samples for whole rock and trace element geochemical analysis.  A database of xxx analyses from Aricheng South, Aricheng North, and Accori were provided to me for geochemical classification, characterization, and comparison with the limited samples I submitted for analysis.  The purpose of my sample suite was to characterize geochemical signatures from the initiation of K-metasomatism to albitization and Zr-U-mineralization.  The data provided by U3O8 Corp. and analyses of rocks I submitted have been plotted on a series of trace element, REE plots, as well as binary plots used to illustrate albitization of K-spar and increase of U-content with albitization.

Trace Element Plots:

The rocks I submitted for analysis were plotted on the MORB normalized diagram of Bevins et al., 1984.

Blocky K-spar over fabric

Initiation of K-spar growth

Complete albitization

Initiation of Na-metasomatism

Zr-U-mineralized sample

The spider plots entitled “initiation of K-spar growth” and “blocky K-spar over fabric” show similar signatures with a positive Th anomaly, and negative Ta, Sr, Ti anomalies.  The spider plots entitled “initiation of Na-metasomatism” and “complete albitization” share similar signatures with a strong negative K2O, Sr, TiO2 anomalies.  The “Zr-U-mineralized sample” shows strong negative K2O anomaly, and a very strong positive Zr anomaly.  The marked Ti anomaly may be related to concentration in late magnetite/hematite??

The data was also plotted on the condrite normalized multi-element diagram of Wood et al., 1979b.

Zr-U-mineralized sample

Complete albitization

Initiation of Na-metasomatism

Blocky K-spar over fabric

Initiation of K-spar growth

The data is also presented here on the REE condrite normalized plot of Nakamura, 1974.

Zr-U-mineralized sample

Initiation of Na-metasomatism

Initiation of K-spar growth

Blocky K-spar over fabric

Complete albitization

The following multi-element plots were compiled using the dataset provided by U3O8 Corp.  The analyses pertain to prospective areas of mineralization (ARS, ARN, ACCORI porspects).

Bevins et al, 1984

ARN SAMPLES

Bevins et al, 1984

ACCORI SAMPLES

Bevins et al, 1984

ARS SAMPLES

Wood et al., 1979b

ACCORI SAMPLES

Wood et al., 1979b

ARN SAMPLES

Wood et al., 1979b

ARS SAMPLES

The company dataset was also plotted on the multi-element plot of Wood et al., 1979b on a prospect basis (ARS, ARN, and ACCORI)

The company dataset was also plotted on the multi-element plot of Nakamura, 1974 on prospect basis (ARS, ARN, and ACCORI).

Nakamura, 1974

ACCORI SAMPLES

Nakamura, 1974

ARN SAMPLES

Nakamura, 1974

ARS SAMPLES